Industrial 4+2 Full Gigabit Managed PoE Switch

(4*1000M PoE+2*1000M SFP)

| Model | 600WP-4G-2S |
| --- | --- |
| Product | Industrial 4+2 Managed PoE Switch |
| Downlink Port | 4*10/100/1000M Port 1~2 support 24V/48V PoE(single port power MAX 30W) Port 3~4 support BT PoE(single port power MAX 90W) |
| Uplink Port | 2*1000M SFP |
| Console Port | 1 |
| Reset Key | 1 |
| Power Input | 2 group of independent power input SW:DC12-57V PoE:DC37-57V |
| Environment Detection | 1 group of input detection of environment temperature and humidity |
| Function Interface(only for enhancement model) | 1 switch for quick formation of ring network 2 group of independent power input, having under- and over-voltage protection 1 group of wide voltage alarm input at a range of DC5-57V 1 group of alarm output and relay output, can define various custom condition trigger 1 group of environmental temperature and humidity detection, can make real-time monitoring of the environment Real-time system of crucial voltage monitoring and alarm output function Real-time system temperature,environment temperature,humidity and alarm output function Real-time detection and display of input voltage,current,power Multiple custom port control and intelligent management |
| Network protocol | IEEE 802.3x IEEE 802.3、IEEE 802.3u、IEEE 802.3ab、IEEE 802.3z IEEE 802.3ad IEEE 802.3q 、IEEE 802.3q/p IEEE 802.1w、IEEE 802.1d 、IEEE 802.1S IEEE 802.3z 1000BASE-X STP(Spanning Tree Protocol) RSTP/MSTP( Rapid Spanning Tree Protocol) EPPS ring network protocol EAPS ring network protocol hasivo 802.3AF/AT/HiPoE/BT hasivo AUTO 24V/48V PSE |
| Port Specification | 1-4 port 10/100/1000BaseT（X）auto detection，AUTO-MDI/MDI-X |
| Transmission Mode | Store and Forward(full wirespeed) |
| Bandwidth | 56Gbps |
| Packet Forwarding | 14.44Mpps |
| MAC | 8K |
| Buffer | 4.1M |
| Transmission Distance | 10BASE-T ： Cat3,4,5 UTP(≤250 meter) 100BASE-TX ： Cat5 or later UTP(≤100 meter) 1000BASE-TX ： Cat6 or later UTP(≤1000 meter) 1000BASE-SX:62.5μm/50μm MMF(2m~550m) 1000BASE-LX:62.5μm/50μm MM(2m~550m) or 10μm SMF(2m~5000m) BIDI supported |
| FLASH | 16M |
| LED Indicator | 5 6:(SFP LED) PoE Port 1-2（Green=48V；Orange=24V）;Link port(Green=LINK LED） Port 3-4(Green=LINK LED；Orange=POE LED） PWR:( Power/system LED) P1:(Master power LED) P2:(Slave power LED) HW:(Ring network ON LED) |
| Power Consumption | Standby power consumption≤5W; Network Full-load power consumption≤10W； Full load power consumption≤250W |
| Operating Temperature/Humidity | -40～+80°C；10%～90% RH No coagulation |
| Storage Temperature/Humidity | -40～+85°C；5%～95% RH No coagulation |
| Lightning Protection Level | 6KV 8/20us；8KV ESD IP40 |
| Product Size/Packing Size（L*W*H） | 157mm*115mm*40mm 310mm*210mm*60mm |
| N.W/G.W（kg） | 0.82/1.1kg |
| Installation | DIN guide rail type(additional wall hangers) |
| Certificate | CE mark, commercial；CE/LVD EN60950； FCC Part 15 Class B；RoHS； |
| Warranty | Whole device for 1 year(Accessories not included) |

| Software Function |  |
| --- | --- |
| The followings are main functions not included in all the functions.Please contact with us for more other information! We support software development for customization requirement. |  |
| Monitoring Platform | Quick formation of ring network System temperature monitoring and alarm External temperature and humidity monitoring and alarm Input power monitoring and alarm System each channel key voltage monitoring and alarm System total current,total power monitoring and alarm All-Port network monitoring and alarm Support one group input and output signal monitoring and alarm |
| Network Protocol | IEEE 802.3x IEEE 802.3、IEEE 802.3u、IEEE 802.3ab、IEEE 802.3z IEEE 802.3ad IEEE 802.3q、IEEE 802.3q/p IEEE 802.1w、IEEE 802.1d、IEEE 802.1S、IEEE 802.1X |
| MAC | Support 16K MAC address,auto aging and learning |
| VLAN Configuration | Support port-based VLAN Up to 4096 VLANs Support Voice VLAN, can configure Qos for voice data 802.1Q |
| Spanning Tree | STP(Spanning tree protocol) RSTP/MSTP(Rapid spanning tree protocol) EPPS/EAPS(Ring network protocol) 802.1x |
| Port Aggregation | Support 8 groups of aggregation, each group support up to 8 ports; |
| Port Mirroring | Support many-to-one port mirroring |
| Loop Guard | Support loop protection, real-time detection, quick alram, concise location, Intelligent blocking, automatic recovery |
| Port Isolation | Support downlink ports isolate from each other and communicate with uplink port as well |
| Flow Control | Half duplex based on Back pressure; Full duplex based on PAUSE frame |
| Speed Limitation | Bandwidth management based on port input and output |
| Multicast Control | IGMPv1/2/3 and MLDv1/2 Snooping GMRP protocol registration Multicast address management, multicast VLAN, multicast routing port, static multicast address |
| DHCP | DHCP Snooping |
| Storm Suppression | Support unknown unicast, multicast , unknown multicast, broadcast type storm suppression Storm suppression based on bandwidth adjustment and storm filtering |
| Security | Support User port+IP address+MAC address ACL based on IP, MAC Support security properties of number of MAC address based on port |
| QOS | 802.1p port queue priority algorithm Cos/Tos,QOS remark WRR(Weighted Round Robin), weighted priority rotation algorithm WRR、SP、WFQ priority scheduling modes |
| Cable Sequence | Auto-MDIX; auto detection on straight-through and cross-over cable |
| Negotiation Mode | Port support auto negotiation function(self-negotiation transmission rate and duplex mode) |
| System maintenance | Upgrade package upload System log viewing WEB recovery factory configuration |
| PoE Function (only for PoE models) | Port switch Schedule arrangement, timing switch Voltage, current, power display System total power display and configuration PoEdog: support auto reset, timing reset and ip binding reset when device failed Port power configuration(AF.AT HiPoE.BT) Indication of over power Port PoE type configuration(48V/24V power supplying) |
| Network Management | WEB interface management CLI management based on Telnet、TFTIP、Console SNMP V1/V2/V3 management RMONV1/V2 management RMON management |