GP5801-02

GPON 2 Port OLT

# Product Overview

The GP5801-02 is a highly integrated, medium-capacity GPON OLT designed for carriers, ISPs, enterprises, and campus applications. It complies with ITU-T G.984/G.988 technical standards, offering excellent openness, strong compatibility, high reliability, and comprehensive software features. It is widely applicable in network construction for FTTH access by carriers, VPNs, government and enterprise campuses, and campus networks.

The GP5801-02 is a compact 1U-height device, making it easy to install and maintain while saving valuable space. Leveraging industry-leading technology, it offers an efficient GPON solution with optimized bandwidth utilization and robust Ethernet service support, enabling operators to deliver reliable service quality to users. The device also supports mixed networking with various types of ONUs, significantly reducing costs for operators.

# Product Interface

GP5801-02

- 2 * GPON port - 2 * 10/100/1000M Base-T - 2*1000M Base-X SFP/2*10GE SFP+ - 265 * 180 * 40mm

# Key Features

- Complies with ITU-T G.984/G.988 technical standards, ensuring strong interoperability.

- Offers rich Layer 2/3 switching features and flexible management options.

- Supports various link redundancy protocols such as FlexLink, STP, RSTP, MSTP, ERPS, and LACP.

- Supports routing protocols including RIP, OSPF, BGP, ISIS, and IPv6.

- Provides redundant power backup.

- Supports terminal power-off alarm notifications.

# Hardware Specification

| Attributes | GP5801-02 |
| --- | --- |
| Interface | 2 * GPON port; 2 * 10/100/1000M Base-T; 2*1000M Base-X SFP/2*10GE SFP+; Dual power supply optional |
| Exchange Capacity | 51.5Gbps |
| Packet Forwarding Rate | 59.7Mpps |
| Memory and Storage | Memory: 32MB; Storage: 512MB |
| Management Port | 1*Console Port; 1*Management Port |
| PON | Complies with ITU-T G.984/G.988 standards 20 km physical differential distance, 60 km logical distance 1:128 maximum splitting ratio Standard OMCI management features Flexibly compatible with any brand of ONT Supports batch software upgrades for ONUs |
| Weight | ≤2kg |
| Fan | 2*Fixed Fans |
| Power | AC: 100～240V, 47/63Hz； DC: 12V |
| Power Consumption | ≤30W |
| Dimensions (W *D *H) | 265mm*180mm*40mm |
| Environment Temperature | Working Temperature：-10℃～55℃ Storge Temperature：-40℃～70℃ |
| Environment Humidity | Operating Humidity: 10%～95% (non-condensing) Storage Humidity: 10%～95% (non-condensing) |
| Environment Friendly | China RoHs,、EEE |

# Software Specification

| Attributes | GP5801-02 |
| --- | --- |
| Vlam | Supports 4K VLAN 802.1Q Supports dual-tag VLAN functionality, including static QinQ based on ports & flexible QinQ |
| MAC | Supports 16K MAC address table entries Supports static MAC address configuration Supports black-hole MAC address filtering Supports port-based MAC address limits |
| Ring Network Protocol | Supports STP/RSTP/MSTP Supports ERPS Ethernet Ring Protection Switching Supports Loopback-detection port loopback detection |
| Port Control | Support two-way bandwidth control Support port storm suppression Support 9K Jumbo ultra-long frame forwarding |
| Port Aggregation | Support static link aggregation Support dynamic LACP Each aggregation group supports a maximum of 8 ports |
| Mirroring | Mirroring |
| ACL | Supports standard and extended ACLs Supports time-based (Time Range) ACL policies Provides flow classification and definition based on IP packet header information such as source/destination MAC address, VLAN, 802.1p, ToS, DSCP, source/destination IP address, L4 port number, and protocol type |
| Qos | Support flow rate limiting function based on custom business flow Supports mirroring and redirection functions based on custom business flows Support priority marking based on custom service flow, support 802.1P, DSCP priority Remark capability Support port-based priority scheduling function Support queue scheduling algorithms such as SP/WRR/SP+WRR |
| Safety | Support user hierarchical management and password protection Supports IEEE 802.1X authentication/Centralized MAC address authentication Support Radius & TACACS+ authentication Support MAC address learning limit Support blackhole MAC functionality Supports SSH 2.0, providing a secure encrypted channel for user login Supports SNMP v3 encrypted management Supports port isolation, with hardware isolation of messages between ports Supports broadcast message rate limiting Supports IP Source Guard, automatically creating an IP+MAC+port+VLAN binding table, and manual binding of table entries Supports automatic suppression of ARP flooding attacks and ARP spoofing protection Supports protection against various CPU-targeted DoS attacks and virus attacks at the control plane |
| Layer 3 Routing | Supports ARP learning and aging Supports creating Layer 3 VLAN interfaces and configuring static IPv4/IPv6 addresses Supports static route configuration Supports RIP/OSPF protocol functions Supports VRRP protocol functions |
| System Management | Supports Console/Telnet/SSH 2.0 command-line configuration Supports configuration management via web browser Supports FTP and TFTP file upload/download management Supports SNMP v1/v2/v3 Supports RMON Supports SNTP clock synchronization Supports system operation logs Supports LLDP (Link Layer Discovery Protocol) for neighbor device discovery Supports 802.3ah Ethernet OAM (Operations, Administration, and Maintenance) Supports RFC 3164 Syslog functionality Supports Ping and Traceroute |

# Purchase Info

| Product Name | Product Description |
| --- | --- |
| GP5810-02 | 2 * GPON port; 2 * 10/100/1000M Base-T; 2*1000M Base-X SFP/2*10GE SFP+; Dual power supply optional |