GPON OLT Series

Datasheet

1.Product overview

- GPON OLT 8PON are highly integrated, medium-capacity GPON OLT for operators, ISPs, enterprises and park Application. The product follows the ITU-T G.984/G.988 technical standard，The product has good openness, strong compatibility, high reliability, and complete software functions. It can be widely used in operators' FTTH access, VPN, government and enterprise park access, campus network access, ETC.

- They are only 1U in height, easy to install and maintain, and save space. Supports mixed networking of different types of ONU, which can save a lot of costs for operators.

- GPON OLT 8PON

2.Main Feature

- Rich Layer 2/3 switching features and Flexible management methods

- Support multiple link redundancy protocols such as Flex-Link/STP/RSTP/MSTP/ERPS/LACP

- Support RIP、OSPF、BGP、ISIS and IPV6

- Safe DDOS and virus attack protection

- Support Power redundancy backup，Modular power supply(8/16PON)

- Support power failure alarm

- Type C management interface(8/16PON)

3. Hardware Features

| - Attributes | - GPON OLT 4PON | GPON OLT 8PON |  | GPON OLT 16PON |
| --- | --- | --- | --- | --- |
| Exchange capacity | 103Gbps | 118Gbps |  | 156Gbps |
| Packet forwarding rate | 76.632Mpps | 87.792Mpps |  | 116.064Mpps |
| Memory and storage | memory：7168M, storage：2048M |  |  |  |
| Management port | Console |  | Console， Type C |  |
| Port | 4*GPON Port，4*10/100/1000M Base-T,4*1000M Base-X SFP/4*10GE SFP+ | 8*GPON Port，4*10/100/1000M Base-T,4*1000M Base-X SFP/4*10GE SFP+ |  | 16*GPON Port，8*10/100/1000M Base-T,4*1000M Base-X SFP/4*10GE SFP+ |
| weight | 3.5kg | 5kg |  | 6.5kg |
| fan | Fixed fans（three fans） |  | Fixed fans（four fans） |  |
| power | AC： 100～240V 47/63Hz； DC： 36V～75V； |  |  |  |
| Power consumption | MAX：50W |  | 75W |  |
| environment friendly | China ROHS EEE |  |  |  |
| Dimensions (Width * height * depth) | GPON OLT 4PON: 440mm*44mm*200mm GPON OLT 8PON: 440mm*44mm*260mm GPON OLT 16PON: 440mm*44mm*320mm |  |  |  |
| Environment temperature | Working Temperature：-10℃～55℃ Storge temperature：-40℃～70℃ |  |  |  |
| environment humidity | Operating humidity: 10%～95% (non-condensing) Storage humidity: 10%～95% (non-condensing) |  |  |  |

4. Software Features

| - Attributes | - GPON OLT 4PON | GPON OLT 8PON | GPON OLT 16PON |
| --- | --- | --- | --- |
| PON | Comply with ITU-TG.984/G.988 standard 20KM transmission distance 1:128 Max splitting ratio Standard OMCI management function Open to any brand of ONT ONU batch software upgrade |  |  |
| VLAN | Support 4K VLAN Support VLAN based on port, MAC and protocol Support dual Tag VLAN, port-based static QINQ and fiexible QINQ |  |  |
| MAC | 16K Mac address Support static MAC address setting Support black hole MAC address filtering Support port MAC address limit |  |  |
| ring network protocol | Support STP/RSTP/MSTP Support ERPS Ethernet ring network protection protocol Support Loopback-detection port loopback detection |  |  |
| port control | Support two-way bandwidth control Support port storm suppression Support 9K Jumbo ultra-long frame forwarding |  |  |
| Port aggregation | Support static link aggregation Support dynamic LACP Each aggregation group supports a maximum of 8 ports |  |  |
| Mirroring | Support port mirroring Support stream mirroring |  |  |
| ACL | Support standard and extended ACL Support ACL policy based on time period Provide flow classification and flow definition based on IP header information such as source/destination MAC address, VLAN, 802.1p, TOS, DSCP, source/destination IP address, L4 port number, protocol type, etc. |  |  |
| QoS | Support flow rate limiting function based on custom business flow Supports mirroring and redirection functions based on custom business flows Support priority marking based on custom service flow, support 802.1P, DSCP priority Remark capability Support port-based priority scheduling function, support queue scheduling algorithms such as SP/WRR/SP+WRR |  |  |
| Safety | Support user hierarchical management and password protection Support IEEE 802.1X authentication Support Radius&TACACS+ authentication Support MAC address learning limit, support black hole MAC function Support port isolation Support broadcast message rate suppression Support IP Source Guard Support ARP flood suppression and ARP spoofing protection Support DOS attack and virus attack protection |  |  |
| Layer 3 | Support ARP learning and aging Support static route Support dynamic route RIP/OSPF/BGP/ISIS Support VRRP |  |  |
| System management | CLI、Telnet、WEB、SNMP V1/V2/V3、SSH2.0 Support FTP,TFTP file upload and download Support RMON Support SNTP Support system work log Support LLDP neighbor device discovery protocol Support 802.3ah Ethernet OAM Support RFC 3164 Syslog Support Ping and Traceroute |  |  |

5.Purchasing Information

| Product name | Product description |
| --- | --- |
| GPON OLT 8PON | 8*PON port, 4*10GE/GE SFP +4GE RJ45 uplink port, Dual power with optional |